# Sumo Robot — Design Specification

Summary of all decisions made during the design planning session.

## Component & Decision Table

| Category | Decision | Dimensions |
| --- | --- | --- |
| Robot Shape | Rectangle | 150 × 150mm footprint, 10mm height |
| Motors | Pololu micro metal gearmotor 1:50 6V HP × 6 | 12 × 35.37mm each |
| Motor Arrangement | 3 per side, horizontal mount | — |
| Wheels | Try 10mm → 12mm → 15mm diameter | 23–25mm width |
| Blade | Steel, 30° angle, full width | 150mm wide, ~17.3mm wall depth |
| Microcontroller | STM32G431 LQFP32 | 7 × 7mm |
| Motor Drivers | Separate boards × 6 | 14 × 21.57mm each |
| IR Sensors | VL53L4CD, front only, vertical mount | 2.4 × 2.4mm each |
| IR Sensor Count | TBD (2, 3 or 4) | — |
| IR Sensor PCB | Separate, connected via FFC cable | TBD |
| Battery | LiPo 1S, 502030, ~150mAh | 5 × 20 × 30mm |
| Battery Connector | XT30 → wires soldered to PCB | ~10 × 15mm |
| Programming Port | 4-pin SWD header | ~10 × 2.5mm |
| On/Off Switch | TBD (finding on AliExpress) | TBD |
| PCB Architecture | Main PCB + separate motor driver boards + IR sensor PCB via FFC | — |
| Outer Walls | 3mm thick, 3 sides | — |

Pending Decisions

• IR sensor count (2, 3 or 4)

• On/off switch model (searching AliExpress)

Next Steps

• Watch Altium Designer tutorials (Phil's Lab / Altium YouTube channel)

• Begin schematic design in Altium Designer

• Finalize remaining pending decisions